Pop3-5.5 WP TMO 版本制作与发布

| File Status： [√] Draft [ ] Released [ ] Modifying | File Identification： | Pop3-5.5 WP TMO 版本制作与发布 |
| --- | --- | --- |
|  | Current Version： | V0.1 |
|  | Author： | 高继泳 |
|  | Completion Date： | 2015.11.16 |

# WP Env setup prerequisites(PC)

- sequence install （\\172.24.220.210\share\jrd-tool）

- Microsoft-win10 include ak/wdk/wpdk/sdk and so on releation tools pls      follow readme

- Arm complie tool RVCT5

- HEXAGON_Tools dev by qualcomm and will update according the BSP version

- Thirdparty  tools

- Qualcomm include download/debug tools

- Android_USB_Driver_Q_7.0.0.0_2014_10_16_All_in_One_noinstall

- for win10 double check end named env20150811.txt

- need replace MSBuild.rar to yours C:\Program Files (x86)

- copy \\172.24.220.210\share \jrd-tool\microsoft-win10\10160\MobileOS

all the sub folders to your computer C:\Program Files (x86)\Windows Kits\10

- Install the following cert, registry and run cmdline, refer to \Kits\HLK\StrongNameSigning\

Testroot.cer

testroot-SHA2.cer

StrongNameBypass.reg,

WOW64STrongNAmeBypass.reg

- Install OEM test certificates by running the script InstallOEMCerts.cmd (this is a one-time step). C:\Program Files (x86)\Windows Kits\10\Tools\bin\i386

- replace a file use \\172.24.220.210\share\WindowsDriver.Default.props when you use VS to complier driver

- replace bat file abut VS read register \\172.24.220.210\share ( VCVarsQueryRegistry.bat vsvars32.bat ) to C:\Program Files (x86)\Microsoft Visual Studio 12.0\Common7\Tools

- merge register about SDK 10 \\172.24.220.210\share\SDK.REG

for complier driver can not find /warning.h error

# How to get source code from git

You can get code from our server

- Creat your local folder in your computer just as D:\WP8909

- Entry the folder cd /d D:\WP8909

- git clone git@172.24.61.94:winphone/amss_8909.git

- after download finish ,your need to checkout code branch

- git checkout amss-pop3-5.5-dev

- make sure you can see the source code as follow

Note：

You also can do following bash script to download code

# Update MSFT AK

The latest MSFT AK is located at \\172.24.220.121\MSFT_AK

It includes folders as follow

delete corresponding folders under C:\Program Files (x86)\Windows Kits\10, Before copy these folders to C:\Program Files (x86)\Windows Kits\10.

# How to compile your code

在D:\WP8909\amss_8909\modem_proc\mcfg\mcfg_gen\generic\NA\TMO目录下，打开MCFG_SW_Items_List_Macro.xlsm文件，勾选如下，并点击有红勾的按钮。

Excute the following script to compile subsystem, boot and ACPI:

build-wp8909-1074.cmd –ALL –RELEASE

you can find the script at D:\WP8909\amss_8909\int_tools. Of course, you can specify parameter to compile specify module, such as if you want only compile UEFI, use the command: build-wp8909-1074.cmd –UEFI –RELEASE

detail usage, you can use build-wp8909-1074.cmd –HELP

# Create FFU image

After step-5 , Excute the following script to create FFU image:

create_ffu.bat production|test PhoneFirmwareRevision

The PhoneFirmwareRevision is defined at QWPCT’s Customize Product Details.

The script file at Folder D:\WP8909\amss_8909\WP\prebuilt\8909

# Using QWpct to Package and subsystem

After compile done, maybe you need package mbn/bin/package files to pkg use QWPCT in your project amss_8909\WP\prebuilt\app\Qwpct, right clip icon

Follow the following tips to Package

S1: Qualcomm Chipset

S2: Customize Product Details

The PhoneFirmwareRevision option naming rule, please reference Appendix-A WP Naming Rule

S3 Customize Package Details

S4 Security Customization

OemPKCertificate and SSDKeys reference to follow the ZIP file

If select BLOW FUSES, the sec.dat file will be merge into FFU image. When DUT download this FFU image, it will be fused.

S5: Factory Customizations

Qualcomm option reference to two files：(the wlan.provision don’t need)

S6 Build Customization

S7: click Build to package and subsystem

# According to Checklist to test

After create FFU image, need to perform a simple functional test on the basis of Checklist file.

# Create TAG label at GIT

Please excute the following command on git bash:

git push ssh://jiyong.gao@172.24.61.94:29418/winphone/amss_8909 HEAD:refs/tags/version

the version is version number, such as WP15 for WP15 version.

# update bugZilla PR status

8.1 Firstly following the picture-8.1 set search condition to find PR status’ VERIFIED_SW on BugZilla website for this version. Saving these PR number to pr.txt (the text file will used in perl script).

Picture-8.1

The includes files as follow:

The usage: perl update_newref.pl WPxx ( readme.txt for details).

After excuting the command, the corresponding pr status will become DELIVERY.

# Create WP releaseNotes file

The include files to create releaseNotes file, as follow:

For detail usage, please reference to readme.txt.

how to use the winphone releasenote:

    - modify WP_POP3-5.5.xls ‘s release-date and release-version

1. goto your git directory, run the follow command and save log.txt to the winphone release note script directory.

git log>log.txt

2. save the verified sw prs from bugzilla into pr.txt, copy the pr.txt to the winphone release note script directory.

3.run get_winphone_releasenote_0.pl, the parameter is the commid id you take in last release.

perl get_winphone_releasenote_0.pl 04e12267e6f2f266e1032ab27c03cd9695febfb2

4.run get_winphone_releasenote_1.pl, the parameter is the release version.

perl get_winphone_releasenote_1.pl WPxx

5.run get_winphone_releasenote_2.pl, the parameter is the release version too.

perl get_winphone_releasenote_2.pl WPxx

Now, you can get releaseNotes file for this version.

# Breakdown FFU for NPI

Open , click key icon to enter edit status. There is no password, please click ok directly.

Input FFU image name at ①;

Input FFU image File absolute path at ②;

Then click  to breakdown FFU image.

There need other files:

- Three BootMode files

- dpp.bin

find at amss_8909\WP\prebuilt\8909\WPCODE

the WPCODE is defined at QWPCT tool.

- sec.dat

at amss_8909\WP\prebuilt\8909\ WP1116\common_output\v1

- QCN tar package - fs_image.tar.gz.mbn

How to create fs_image.tar.gz.mbn, please reference to Appendix-B

- prog_emmc_firehose_8909_lite.mbn

at amss_8909\WP\prebuilt\8909\signedmbn

- keystore.dat

at SSDKeys\keys

- Flash.FFU (retail_second 下的Flash.FFU)

- 添加一个空的bin文件，取名为：YWPxxxxx.bin；

例如本项目的WP30版本，则文件取名为YWP01230.bin，WP31版本则命名为YWP01231.bin；以此类推。

# Transfer FFU to teleweb and shared folder

10.1 transfer FFU to TeleWeb

Open FileZilla Tool, add site as follow:

Host: 172.24.220.198

User: sl_jhzhu

Passwd: Aa123456

Under /automount/sw_liv/livraison_securise/Pop3-5.5_WP_TMO, there are four folders:

The appli folder save official version;

The black folder save black version;

The MINI folder save version for NPI;

The tmp fold save tmp version;

10.2 transfer FFU and back up related folders to Shared-folder

The Shared folder is at \\172.24.221.122\d\ ,

Also need to back up related folder. They are:

- amss_8909\WP\prebuilt\8909\firmware

- amss_8909\WP\prebuilt\8909\FMFiles

- amss_8909\WP\prebuilt\8909\ICD_project

- amss_8909\WP\prebuilt\8909\OEMCustomization

- amss_8909\WP\prebuilt\8909\OEMInputTemplates

- amss_8909\WP\prebuilt\8909\signedmbn

- amss_8909\WP\prebuilt\8909\spkg

- amss_8909\WP\prebuilt\8909\WP1116

  - The WP1116 is project name, you define it on QWPCT tool.

# Appendix-A: WP Naming Rule

| WP naming rule |  |  |  |
| --- | --- | --- | --- |
| please find phone info at setting->system->about->device information->more info |  |  |  |
| Model name | ALCATELONETOUCH FierceXL 5055W |  |  |
| OS version | MSFT AK version |  |  |
| fireware version |  |  |  |
| 1077 | 0 | ABCD | EFGH |
| QCT baseline changed when the baseline updated |  | AB: project--01 C: 0--test; 1--production; 2--retail D: QCT signed or not, 0 not signed,1 signed | EF: software version number,09, 10, 11... GH: black version number,01,02,03…. |
| hardware version |  |  |  |
| bootloader | BOOT.BF.3.1.2.c2-00016-M8909AAAAANAZB-1 |  |  |
| Radio software version | MPSS.JO.1.0.c5-00051-M8909EAAAANVZM-1(current value) |  |  |
| Chip SOC version | 8909 |  |  |
| screen size | 720x1280 |  |  |
| MAC address | WIFI mac address |  |  |
| IMEI | IMEI |  |  |

# Appendix-B: Creating fs_image.tar.gz.mbn

文件所在位置：D:\WP8909\amss_8909\WP\prebuilt\8909\firmware

- 注释掉D:\WP8909\amss_8909\modem_proc\core\storage\efs\inc\custefs.example.h 中的宏定义FEATURE_EFS_ENABLE_FACTORY_IMAGE_SECURITY_HOLE；

- 在D:\WP8909\amss_8909\modem_proc\build\ms\cust8909.gen.prodq.h中定义宏FEATURE_EFS_ENABLE_FACTORY_IMAGE_SECURITY_HOLE，格式如下：

#define FEATURE_EFS_ENABLE_FACTORY_IMAGE_SECURITY_HOLE

- 在D:\WP8909\amss_8909\WP\prebuilt\8909\app\Qwpct\scripts\Factory\Factory.psm1中，将函数GenerateEFSTar注释，这样Qwpct工具就不会自动生成tar包。

- 执行以下命令：

cd D:\WP8909\amss_8909\modem_proc\build\ms

build.cmd 8909.gen.prod –k

- 执行Qwpct,最后一项只选最后一项

- 将D:\WP8909\amss_8909\WP\prebuilt\8909\signedmbn\8909的qcdsp1v28909.mbn和qcdsp28909.mbn拷贝到手机的system32目录中。

- 打开手机diag口，并打开QPST，查看手机是否连接上，

- 使用如下命令，生成fs_image.tar.gz:

perl efsreadimage.pl –z

- 将上步创建的压缩文件copy到D:\WP8909\amss_8909\WP\prebuilt\8909\firmware

先备份D:\WP8909\amss_8909\WP\prebuilt\8909\signedmbn

打开Qwcpt，设置如下：

执行完后，可以在D:\WP8909\amss_8909\WP\prebuilt\8909\signedmbn中获取signed过的fs_image.tar.gz.mbn

# Appendix-C: How to downlaod FFU image

方法1：进入闪电模式后刷机

先短按power键开机，然后立即长按VOL+键，直到出现如下图所示，即表明进入闪电模式。然后使用如下命令：

ffutool –flash ffu.FFU

方法2：开启9008端口后刷机

手机关机后，直接用强制下载线连接手机。即可在设备管理器中查看到，如下图：

然后使用如下命令：

emmcdl –p COM13 –f prog_emmc_firehose_8909_lite.mbn –ffu ffu.FFU

# Appendix-D: how to backup FFU image

在D:\Building_Version下有个backup_black.cmd脚本，用法如下：

backup_black.cmd WP30 WP3000 production|test|retail

Note:

对于正式版本的版本，使用D:\Building_Version下的backup_to_Server.cmd脚本，用法如下：

backup_to_Server.cmd WP30 WP3000 production|test|retail WPCODE